British Marine Loss Prevention Surveys - A Guide for Surveyors Aligned to the 2026-2030 Vessel Risk Survey Form (Core)

Updated edition prepared to align guidance with the current survey form and reporting workflow.

Read these notes before carrying out the survey. If further advice is needed, contact the British Marine Loss Prevention Department. This version keeps the existing structure of the guide, removes the D - Dangerous reporting category, updates the photographs guidance, and updates the risk analysis guidance to match the current form, which does not require probability, consequence, or calculated risk factor entries. Scope and limitations of this survey This survey is a loss prevention risk assessment carried out on behalf of British Marine for insurers’ purposes. It is intended to identify conditions, practices, or management factors which may reasonably lead to an incident or claim and to provide a professional judgement of the associated risk at the time of attendance. This survey does not constitute: - a statutory, class, or flag state inspection; - certification of compliance with international conventions, flag requirements, or classification rules; or - confirmation that the vessel is seaworthy or fit for any specific trade or service. Surveyors should base all findings strictly on what is observed, demonstrated, or evidenced during the attendance. No assurance is given, and none should be implied, that conditions will remain unchanged following the survey.

# Contents

1. General Procedures and Administration

2. Reporting

3. Photographs

4. Risk Analysis

5. Hazard Description

6. Perceived Risk

7. Probability

8. Consequence

9. Risk Regions

10. Risk Reduction (and recommendations for action)

11. Completing the Risk Analysis Report Form

12. Follow Up Surveys

13. Safety Audits of Passenger Vessels

14. Towage Approvals

15. Voyage Approvals

16. Reporting for Non-Standard Surveys

17. Surveyors Optional Checklist

# General Procedures and Administration

British Marine may instruct surveys for P&I cover, Hull & Machinery cover, or both. The surveyor should concentrate the report on matters that are relevant to the insured risk while recognising that many areas overlap, including navigation, fire protection, management, machinery condition, and operational controls.

A survey is normally expected to be completed within a day. If a longer attendance is considered necessary, the British Marine Loss Prevention Department should be advised immediately.

Items should be tested wherever appropriate, always with the prior knowledge and cooperation of the ship's staff. Where an area is clearly well maintained and operational, time should be focused on the more important items or those whose condition gives cause for concern.

Passenger-carrying vessels should be subject to a safety audit of passenger arrangements. Towage approvals should include a review of the tug, tow, towage arrangement, voyage planning, and weather routing. Voyage approvals should address the vessel's suitability and preparedness for the intended passage.

Ballast tanks should be pressed up and holds inspected on general cargo and bulk carriers where practicable. If not possible, this should be recorded in the report. For P&I surveys, hatch covers should also be tested where applicable and feasible.

Enclosed spaces should not be entered unless the surveyor is completely satisfied that the space is safe for entry.

Surveyor appointment

Surveyors are appointed by British Marine in accordance with its surveyor appointment arrangements and instructions. Surveyors should undertake only the scope of work expressly instructed and should refer any uncertainty regarding scope, access, or reporting expectations to the British Marine Loss Prevention Department without delay.

# Reporting

The report consists of the vessel and survey particulars, the overall and sectional risk assessment, the hazard table, the photographs appendix, and any additional comments. The Master or other representative should be made aware of material findings, and serious defects or deficiencies should be clearly emphasised during the attendance.

A typed Microsoft Word version of the report, not PDF, should be e-mailed to the British Marine Loss Prevention Department as soon as possible after the survey, normally within one working day / 24 hours for the provisional submission, followed by the full final report within the required timescale.

The report should include the necessary photographs and any relevant supporting material. The invoice and other supplementary material may be sent later if required. There is no need to send the aide-memoire checklist or copies of the vessel's certificates unless specifically requested. A copy of the crew list should not be taken or sent.

The summary ratings in the current form use the following categories only: Excellent, Good, Satisfactory, and Poor.

• If a section is rated Satisfactory or Poor, a brief comment should be provided in Section 3 of the form.

• If a section is rated Poor, the rating should normally be supported by at least one related hazard in Section 4 and by appropriate photo references in Section 5.

• If any urgent issue is identified that could reasonably result in injury, pollution, or an operational incident in the near term, British Marine should be contacted immediately and the matter should be stated clearly at the start of Section 2.2 with the relevant hazard and photo references.

The General Overview / Overall Narrative Summary should be concise, factual, and based on what was observed or demonstrated during the survey. Additional comments may be used to help British Marine understand the basis for the assessment. These comments should normally remain concise unless the vessel is notably sub-standard.

# Authority and reporting boundaries

Surveyors are engaged solely to report observations and professional risk opinions. Surveyors must not:

- indicate or imply that insurance cover may attach, continue, or be varied;

- agree, suggest, or confirm that deficiencies, hazards, or recommendations are acceptable to insurers; or

- provide assurances to owners, managers, or masters regarding underwriting decisions, warranties, or conditions of cover.

Decisions regarding acceptability of risk, imposition or removal of conditions, and confirmation of cover remain entirely with British Marine and its Underwriting function.

# Photographs

The current form requires a representative set of at least 20 clear photographs, with additional photographs for each hazard where applicable. Photographs should support the narrative and recorded hazards rather than replace them.

• Provide general coverage photos of the vessel, including external hull views where practicable, main deck areas, bridge, machinery spaces, accommodation overview, and relevant cargo, towage, fishing, or passenger arrangements as applicable.

• Provide clear photographs of each material hazard item where practicable and safe to do so.

• Where higher risk is identified, or where several sections are rated Poor, more than the minimum number of photographs is expected.

• Captions should follow the form requirement: Photo XX: [Area] - [What / defect] - [Why it matters].

• Photo numbers should be referenced in Section 3 comments where relevant and in Section 4 for related hazards.

• A separate photo index is not required if the appendix is clearly captioned and cross-referenced.

The aim is to provide British Marine with enough visual evidence to understand the general condition of the vessel and the significance of the hazards recorded.

# Risk Analysis– Guidance for Surveyors

Loss prevention surveys have traditionally focused on the physical condition of the vessel and, in some cases, the way in which it is managed or operated. While this remains important, British Marine requires surveyors to focus primarily on risk and to make professional judgements on our behalf as insurers, whether for P&I cover, Hull & Machinery, or both.

The surveyor is required to consider the vessel and its operation as observed at the time of attendance and to identify those conditions, practices, or management factors that may reasonably lead to an incident, and therefore to a claim.

Risk is not limited to structural or machinery defects. Human, procedural, and organisational factors are equally relevant, particularly for P&I exposures such as crew injury, pollution, collision, cargo damage, passenger claims, towage failure, and third-party liability.

Each identified hazard must:

- clearly describe the unsafe condition or practice,

- explain why it matters from an insured-risk perspective, and

- include a practical corrective action aimed at reducing the risk.

Unjustified or disproportionate defect reporting is not acceptable. Findings must be fact-based, proportionate, and relevant to insured risk.

Use of risk concepts

References to probability, consequence, and risk significance in this guide are provided solely to assist the surveyor’s internal professional judgement when determining the emphasis, urgency, and clarity of reported hazards.

These concepts are not to be recorded, scored, calculated, or explicitly referenced in the completed survey report narrative, tables, or ratings, as the current survey form does not require or provide for their inclusion.

# Hazard Description

A hazard is a situation, condition, or practice that represents a potential source of harm, accident, or damage.

Examples include:

- Defective or deteriorated structure or equipment due to corrosion, damage, or lack of maintenance.

- Missing, inoperative, or poorly maintained safety equipment (e.g. guards, firefighting systems).

- Unsafe, informal, or poorly controlled working practices.

- Inadequate procedures, supervision, training, or safety culture.

It is essential that the hazard itself is described, not merely a defect. The wording should make clear what is unsafe and why it matters.

# Perceived Risk

For each hazard, the surveyor must explain why it should be addressed, by linking it to the most likely insured risk.

Examples:

- Hazard: Significant corrosion or cracking in primary structural members
Risk: Structural failure, water ingress, pollution, cargo loss, loss of life, wreck removal exposure.

- Hazard: No enclosed space entry procedures or controls
Risk: Crew fatality or serious injury.

The focus should be on the most credible and material outcomes, not every conceivable scenario.

# Likelihood (Probability) – Guidance Only

When judging the seriousness of a hazard, it is helpful to consider how likely it is to cause or contribute to an incident:

- High: Almost certain to cause or contribute

- Probable: Likely

- Possible: Not likely, but credible

- Improbable: Unlikely, but not impossible

This judgement should influence how strongly the issue is emphasised in the report and how urgent the recommended action is.

| Probability | Code | Guidance description |
| --- | --- | --- |
| High | 4 | Almost certain to cause or contribute |
| Probable | 3 | Likely |
| Possible | 2 | Not likely |
| Improbable | 1 | Not impossible but most unlikely |

# Severity (Consequence) – Guidance Only

The potential severity of an incident should also be considered:

- Catastrophic: Loss of life, total loss, or widespread severe pollution

- Major: Serious injury, major damage, or severe pollution

- Moderate: Injury requiring medical attention, or significant localised damage

- Minor: Minor injury or limited damage

Severity helps determine whether an issue should materially affect the vessel’s overall risk assessment.

| Consequence | Code | Description |
| --- | --- | --- |
| Catastrophic | 4 | Loss of life, total loss of ship and/or cargo, or Widespread and very severe environmental damage. |
| Major | 3 | Serious injury, major fracture or loss of limb – requires hospitalisation. Severe damage to ship and/or cargo, or severe environmental damage. |
| Moderate | 2 | Injury/illness requiring medical attention; some temporary impairment. Significant damage to ship and/or cargo, or significant but localised environmental damage. |
| Minor/negligible | 1 | Minor injury. Medical expertise not required; no impairment of ability. Minor or inexpensive damage to ship and/or cargo, or none or very limited local environmental damage. |

# Risk Significance (Guidance Only)

Traditionally, combining likelihood and severity helps identify whether a risk would be considered:

- Intolerable: Requires immediate or urgent action

- Significant: Acceptable only in the short term; prompt risk reduction required

- Negligible: Acceptable, though improvement may still be recommended

These concepts remain useful as internal guidance when deciding urgency and emphasis, even though no formal risk category is recorded in the report.

|  |  |  | Pro | bability |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 Improbable | 2 Possible | 3 Probable | 4 High |
| Consequence | 4 Catastrophic | 4 Significant | 8 Significant | 12 Intolerable | 16 Intolerable |
|  | 3 Major | 3 Significant | 6 Significant | 9 Intolerable | 12 Intolerable |
|  | 2 Moderate | 2 Negligible | 4 Significant | 6 Significant | 8 Significant |
|  | 1 Minor | 1 Negligible | 2 Negligible | 3 Significant | 4 Significant |

# Risk Reduction and Recommendations

Risk reduction may be achieved by:

- reducing the likelihood of an incident,

- reducing the severity of the consequences, or

- reducing both.

This may involve improvements to:

- maintenance or engineering condition,

- management or supervision,

- procedures or permit-to-work systems,

- training or crew awareness,

- operational or emergency arrangements.

Recommendations should be clear, practical, and suitably broad, allowing the owner or operator to determine the most appropriate method of implementation.

# Completing the Risk Analysis Report Form

Use the current form as follows:

• Number each hazard item sequentially.

• Refer to the relevant Section 3 area in the Section / Area column.

• Describe the hazard as the unsafe condition or practice, not merely the defect.

• State the P&I risk affected, for example crew, cargo, pollution, collision, passenger, towage, or third-party exposure.

• Record a broad recommended corrective action.

• Use the overall score, narrative, and sectional ratings to reflect your judgement of the vessel's risk profile; do not enter probability, consequence, or calculated risk factor values, as these are not fields in the current form.

# Follow Up Surveys

Follow-up surveys and rectifications

Where a follow-up survey is instructed, the surveyor’s role is limited to verifying the status of previously reported hazards and recording whether corrective actions appear to have been implemented based on observation or evidence provided at the time of attendance.

Surveyors must not independently chase owners, managers, or operators for rectifications, nor provide confirmation that matters are “closed” for insurance purposes, unless expressly instructed to do so by British Marine.

Any uncertainty, partial rectification, or new hazard identified during a follow-up attendance should be reported clearly, with supporting photographs where appropriate.

When supplied with an instruction for a follow-up survey, the original hazard list will normally also be provided. All deficiencies from the previous survey should be re-checked and confirmed as rectified or outstanding.

The priority is to confirm the status of the original hazards. If the surveyor becomes aware of any notable new hazards, these should be brought to the attention of the Master and reported clearly as NEW items with supporting photographs where appropriate.

# Safety Audits of Passenger Vessels

A passenger-vessel safety audit should review the vessel's safety procedures and the crew's ability to respond to an emergency. It should address passenger safety information, signage for muster stations, evacuation routes, lifejacket location, and the practical readiness of the crew.

Testing of the fire detection system, ventilation control, flooding response, and passenger evacuation arrangements should be considered where appropriate. Random inspection and testing of safety equipment and review of maintenance and drill records should also be included.

# Towage Approvals

Towage approvals require the surveyor to assess and certify, so far as instructed, the tug, tow, and towage arrangements, including weather routing and voyage planning, as fit for the intended voyage or employment. The surveyor should make clear to the assured's representative that the surveyor's requirements are to be complied with at all times.

# Voyage Approvals

Voyage approvals require a review of the vessel's suitability and preparedness for the planned voyage. In addition to the surveyor's own judgement and experience, any separately supplied voyage-approval guidance should be completed where instructed.

# Reporting for Non-Standard Surveys

Non-standard surveys, such as voyage approvals, towage approvals, and passenger safety audits, should be reported using the standard form as closely as possible. Use Not Applicable where appropriate and use the overall narrative and summary sections to record matters that do not fit neatly under the standard headings. Separate attachments may be used where diagrams or additional explanation are needed.

# Surveyors Optional Checklist

This checklist is provided as an aide-memoire only and does not form part of the final report. It is not intended to be completed in full or treated as a tick-box exercise.

Surveyors are expected to apply professional judgement to prioritise attendance time and reporting focus on areas that present the most credible and material risk to insured interests, having regard to vessel type, age, trade, and observed condition.

## Ships Certificates and Documents

• Registry, classification, statutory certificates, ISM / ISPS documentation, PSC and flag records, safe manning, recent class status, and any overdue surveys or unresolved recommendations.

## Navigation and Communications

• Bridge equipment, voyage planning, charts / ECDIS, BNWAS, alarms, publications, communications equipment, logbooks, and steering / control changeover procedures.

## Management and Manning

• Standing orders, stability information, emergency response procedures, certificates of competency, language, fatigue, risk assessments, permit-to-work controls, and SMS effectiveness.

## Fire Prevention and Emergency Response

• Fire detection, firefighting appliances, lifesaving appliances, maintenance records, emergency drills, pollution response procedures, and crew awareness of emergency duties.

## Hull, Structure, Machinery, Cargo, Pollution, Personal Safety, and Deck Machinery

• Condition, integrity, access, alarms, pollution controls, cargo / tank / hatch arrangements, enclosed-space controls, lifting gear, mooring arrangements, and supporting records.

# Surveyors Optional Checklist

Note that this checklist is not part of the report

The purpose of this checklist is to provide an aide-memoire for the surveyor.  It is not intended to be comprehensive and the surveyor should use his own experience and opinion to report and assess anything that may be a hazard and lead to a claim.

## Ships Certificates & Documents 	 	 	 	         Check box as appropriate

## All Ships

| Item | Checked |
| --- | --- |
| Registry |  |
| Classification |  |
| International Loadline – Issue/Annual |  |
| Safety Construction - Issue/Annual/Intermediate |  |
| Safety Equipment - Issue/Annual |  |
| Safety Radio |  |
| Hull - Special/Annual/Intermediate |  |
| Machinery - Special/Annual |  |
| Drydock Survey |  |
| Firefighting Appliances |  |
| ISM Safety Management Certificate |  |
| ISM Document of Compliance |  |
| ISPS Certificate |  |
| Enhanced Survey report |  |
| Refrigeration Machinery |  |
| Cargo Gear - Quadrennial/Annual/Renewal |  |
| Intl. Oil Pollution Prevention Cert - Issue/Annual/Intermediate |  |
| Intl. Sewage Pollution Prevention Cert. |  |
| Intl. Air Pollution Prevention Cert. - Issue/Annual/Intermediate |  |
| Anti-Fouling Certificate |  |
| Bunker Civil Liability Cert. (BCLC) |  |
| Marpol Garbage Certificate |  |
| Record of last Port State Control inspection |  |
| Last flag state inspection |  |
| Minimum safe manning document |  |
| Maritime Labour Certificate |  |
| Wreck Removal Certificate (WRC) |  |
| Recent class status report |  |
| Approved Inventory of Hazardous Materials – Part I |  |
| In addition, for Oil, Chemical and Gas tankers, as appropriate |  |
| Noxious Liquid Substances Certificate |  |
| Certificate of Fitness for Dangerous Chemicals in Bulk |  |
| Certificate of Fitness for Liquefied Gases in Bulk |  |
| Civil Liability Cert. (CLC) |  |
| The following should be reported as a hazard: |  |
| Any surveys that are overdue |  |
| Conditions of Class, Memoranda and Recommendations |  |
| Port state control deficiencies not satisfactorily closed |  |
| Flag state inspection deficiencies not satisfactorily closed |  |

## Section 1   -   Navigation & Communications

| Gyrocompass and repeaters |  |
| --- | --- |
| Magnetic compass and deviation calibration curve |  |
| Record of compass error observations |  |
| Radar sets, ARPA |  |
| GMDSS equipment and logbook |  |
| Echo sounder |  |
| Engine order recorder |  |
| Speed and distance measuring equipment |  |
| Shaft revolution indicators |  |
| Rudder angle indicator |  |
| Propeller pitch angle |  |
| Procedure for changeover of steering/engine/thruster control |  |
| Satellite Navigation Receiver |  |
| ECDIS |  |
| Bridge Navigational Watch Alarm System |  |
| Notices to Mariners (Latest edition on board) |  |
| Charts for next passage(s) corrected to date |  |
| Master-Pilot exchange documented |  |
| Berth to Berth voyage planning |  |
| UKC calculation included in voyage plan |  |
| ICS/IMO Bridge Procedures Guide |  |
| Sailing Directions for current voyage (Pilot books) |  |
| Navigation publications, as appropriate |  |
| List of Radio Signals |  |
| List of Lights |  |
| Navtex receiver |  |
| Daylight signalling lamp |  |
| VHF radios |  |
| EPIRB |  |
| AIS |  |
| VDR |  |
| Deck Log Book |  |
| Pyrotechnics & Line throwing appliance |  |
| General alarm system |  |

## Section 2  -  Management & Manning

| Item | Checked |
| --- | --- |
| Company Standing Orders and Instructions |  |
| Master’s / Chief Engineer’s Standing Orders |  |
| Superintendent or head office attendance on board |  |
| Intact Stability Book |  |
| Loading Manual |  |
| Method of calculating stability at all voyage stages |  |
| Bending, torsion moments & shear force calculations |  |
| Record of previous cargoes and loading conditions |  |
| Compliance with applicable Codes of Practice |  |
| Emergency response procedures and effectiveness |  |
| Compliance with Safe Manning Certificate |  |
| Certificates of competency |  |
| Dangerous cargo endorsements |  |
| Adequate experience for cargo & trading pattern |  |
| Knowledge of English |  |
| Common working language on board |  |
| Opinion of on-board management |  |
| Use of risk assessments |  |
| Permit to Work system in use |  |
| Drug & alcohol policy enforced |  |
| Safety Management System (ISM) |  |
| SMS manuals on board and in use |  |
| Master’s understanding of the SMS |  |
| Officers know who the Designated Person is |  |
| Records of non-conformances |  |
| Accident & hazard reports |  |
| Records of corrective actions |  |
| Internal safety audits |  |

## Section 3   Fire Prevention & Emergency Response

| Item | Checked |
| --- | --- |
| Fire hazards identified in all areas |  |
| Control of ventilation, heat & smoke spread |  |
| Fire detection system |  |
| Muster List posted, current & understood |  |
| Firefighting equipment ready for use |  |
| Firefighting servicing documentation |  |
| Lifesaving appliances ready for use |  |
| Lifesaving appliance servicing documentation |  |
| Instructions for FFE & LSA in suitable language |  |
| SOLAS Ch. III maintenance checks & records |  |
| Pollution prevention procedures (SOPEP) |  |
| Emergency drills & exercises |  |
| PA system audible in all manned spaces |  |

## Section 4   -   Hull & Structure

| Item | Checked |
| --- | --- |
| Watertight doors |  |
| Fan flaps & ventilator closures labelled |  |
| Sounding pipes & screwed caps labelled |  |
| Air pipes & gauzes labelled |  |
| Guardrails |  |
| Steps, ladders, gangways & safety nets |  |
| Shell plating, decks & superstructure |  |
| Temporary doublers class-approved |  |
| Date of last steel thickness measurements |  |
| Draught marks & load lines visible |  |
| Extent & seriousness of wastage |  |
| Coating type & condition |  |
| Sacrificial anodes – fitting & wastage |  |

## Section 5 -   Machinery

| Item | Checked |
| --- | --- |
| Ship operates with UMS |  |
| General condition of machinery spaces |  |
| Main engine |  |
| Auxiliaries & generators |  |
| Adequate lighting |  |
| Emergency escape routes clearly signed |  |
| Lift – valid test/maintenance certificate |  |
| Class-approved PMS |  |
| Machinery maintenance records |  |
| Lube oil analysis records |  |
| Insulation tests |  |
| Engine room log |  |
| Alarm & shutdown testing |  |
| Quick-closing valves / remote stops |  |
| OWS working & overboard valve secured |  |
| Sea inlets & discharge valves |  |
| Bilge alarms |  |
| Stern seal |  |
| Piping labelled / colour-coded |  |
| Steering gear |  |
| Adequate spares |  |
| Emergency generator |  |
| Fuel quality testing |  |
| Valves labelled with function |  |
| Floor plates secured |  |

## Section 6   -   Cargoworthiness

### Cargo holds

| Item | Checked |
| --- | --- |
| Fit for intended cargo |  |
| Bilges clean & free of debris |  |
| Bilge suctions & non-returns tested |  |
| Air & sounding pipes pressure tested |  |
| Tanktops & manholes tested & examined |  |

### General cargo ships and bulk carriers

| Item | Checked |
| --- | --- |
| Hatch cover operation |  |
| Condition of hatch covers |  |
| Compression bars & seals |  |
| Securing devices |  |
| Drain channels & non-returns |  |
| Tarpaulins / battens / wedges (if fitted) |  |
| Coaming structure |  |
| Access hatches labelled & sound |  |
| Container fittings on covers |  |
| Watertightness testing (UT preferred) |  |
| Water ingress alarms |  |
| Hold coatings |  |
| Corrosion / scale |  |
| Access ladders & guardrails |  |
| Tween deck covers |  |
| Spar ceiling |  |
| Hold ventilation |  |
| Lighting |  |
| Fumigation procedures |  |

### Refrigerated Cargo Vessels

| ### Item | ### Checked |
| --- | --- |
| Cleanliness & cargo suitability |  |
| Insulation & lagging |  |
| Refrigerant / brine |  |
| Ventilation control |  |
| Temperature monitoring |  |
| Air delivery & return sensors |  |
| Airflow measurement |  |
| CO₂ monitoring |  |
| Humidity recording |  |
| Emergency alarms |  |
| Previous cargo temperature records |  |
| Classification compliance |  |

### Container vessels

| ### Item | ### Checked |
| --- | --- |
| Stowage plan, Class approved |  |
| Lashing manual, Class approved |  |
| Sufficient lashing equipment tested and examined; record maintained |  |
| Sufficient fixed and portable securing devices |  |
| Cell guides and pads |  |
| Officers aware of tier and weight restrictions |  |
| Hazardous cargo separation |  |
| Temperature control facilities and electrical connections |  |
| Reefer sockets |  |
| Reefer monitoring |  |
| Lashing platform condition |  |
| Temporary guardrails available for open hatches |  |

### Passenger and/or RoRo vessels

| Item | Checked |
| --- | --- |
| Fire detection system |  |
| Fixed firefighting system |  |
| Fire control boundaries and doors |  |
| Watertight doors |  |
| Bow, shell and stern doors |  |
| Water ingress and flooding alarms |  |
| CCTV monitoring |  |
| Crew clearly identifiable as such |  |
| Passengers cleared from RoRo spaces before passage commences |  |
| Safety in public spaces |  |
| Galley and food handling and consumption areas |  |
| Safety notices and information for passengers |  |
| Passenger control and assistance in an emergency |  |
| Escape routes clearly marked |  |
| Condition and certificates of ramps and lifts |  |
| Clear separation between passenger and vehicle access |  |
| Vehicle lashings |  |

### Tankers

| Item | Checked |
| --- | --- |
| Safe access to bow |  |
| Emergency towing arrangements |  |
| Technical and operational information |  |
| Awareness of operational parameters of pressure, loading rate, venting etc |  |
| Cargo handling plan and procedures |  |
| Availability of product data sheets |  |
| Implementation of safe operating procedures |  |
| Valve operating system |  |
| Pipeline condition |  |
| Manifold valves, blanks and savealls |  |
| Oily water discharge at manifold ('Marpol' line) |  |
| Venting system and P/V valves |  |
| Seals of ullage ports and tank lids |  |
| Ullaging system |  |
| Level alarms |  |
| Tank washing system |  |
| Tank cleaning guide |  |
| Heating coils |  |
| Pumproom |  |
| Permanent warning signs at entrances |  |
| Rescue equipment immediately available |  |
| Gas detection system |  |
| Firefighting system and equipment |  |
| Lighting |  |
| Ladders and guardrails |  |
| General cleanliness |  |
| Ventilation |  |
| Bilges and bilge level alarm |  |
| Pipework and valves - permanent identification |  |
| Pumps and prime movers |  |
| Cargo pump relief valves; last tes |  |
| Bulkhead seals |  |
| Pump emergency stops |  |
| Stripping pumps |  |
| Instrumentation and controls |  |
| Communications to CCR and ER |  |
| Inert gas system |  |
| Fixed and portable measuring equipment |  |
| Fixed and portable oxygen measuring equipment |  |
| Pressure recording equipment for system and in tanks |  |
| Oxygen levels at generator and in tanks |  |
| Pressure in tanks (random checks) |  |
| Deck water seal |  |
| Alarms |  |
| Chemical tankers |  |
| IMO Ship Type |  |
| Coating compatibility guide |  |
| Cargo compatibility guide |  |
| Cargo handling manual |  |
| Deck tanks |  |
| Protective clothing and SCBA |  |
| Decontamination showers |  |
| Eyewash stations |  |
| Gas carriers |  |
| Cargo tank type and construction |  |
| Permitted temperature and temperature |  |
| Void spaces; inert condition |  |
| Cargo pumps |  |
| Emergency cargo pump |  |
| Tank relief valves |  |
| Instrumentation and recording equipment |  |
| Emergency Shut Down (ESD) Systems |  |

## Section 7   -   Pollution Prevention

| Item | Checked |
| --- | --- |
| Crew awareness |  |
| Oily water separator and instructions for operation |  |
| Entries in Oil Record Book (Part I and Part II) |  |
| Approved Shipboard Oil Pollution Emergency Plan (SOPEP) |  |
| Oil spill control measures |  |
| Emergency response procedures |  |
| Connections, valves, scupper plugs and manifold savealls |  |
| Written procedures sighted and understood by ship's staff |  |
| Named responsible officer and assistants |  |
| Communications between ship and bunker point |  |
| Bunkering procedure, sample and Bunker Delivery Note (BDN) retention |  |
| Fuel changeover procedure |  |
| NOx technical file |  |
| Approved Garbage Management Plan |  |
| Garbage Record Book Entries (Part I and Part II) |  |
| Approved Ballast Water Management Plan |  |
| Ballast water logbook entries |  |

## Section 8   -   Personal Safety

| Item | Checked |
| --- | --- |
| Protective clothing and safe working equipment |  |
| Safe stowage of flammable stores and chemicals |  |
| All gas cylinders safely stowed in sheltered external location |  |
| Are all enclosed spaces clearly identified |  |
| Medical facilities |  |
| Care and security of dangerous/restricted drugs |  |
| Adequate instruction and maintenance manuals in suitable language |  |
| Notices and labels in suitable language |  |
| ISPS related activities, including gangway security and stowaway searches |  |
| Safe access arrangement |  |
| Equipment fitted with necessary safety guards as required |  |
| Appropriately rated insulated matting in use at switchboards |  |
| Portable gas detector available |  |

## Section 9   -   Deck Machinery & Miscellaneous

| Item | Checked |
| --- | --- |
| Deck machinery - including capstans, winches |  |
| Cranes, derricks, wires and ropes |  |
| Swing radius of crane marked or barriers provided |  |
| Mooring ropes, wires, bitts and fairleads |  |
| Windlass and anchor chain condition |  |
| All parts of lifting machinery examined at least annually by competent person |  |
| Record of maintenance and examinations |  |
| Proof load certificates for all derricks and cranes - SWL clearly marked |  |
| Test certificates for all wires |  |
| Condition of blocks, wires and shackles |  |
| Condition of winches and cranes |  |
| Adequate spares |  |
| Pilot ladders |  |
| Accommodation ladders / gangways |  |
| Galley and stores |  |

Final report discipline Guidance notes are for surveyor use. They should not be left embedded in final client-facing report outputs unless British Marine specifically requires them.

British Marine | 30 Fenchurch Street, London EC3M 3BD | lossprevention@britishmarine.com